Managing an archival collection or a high-stakes file room is a bit like being a librarian for time itself—if you don't stay on top of the environment, the environment stays on top of your records.

Here is a streamlined Preservation Checklist designed to keep your "Guide File" collection in pristine condition.

📦 1. Box Labels & Organization

Ensure every container is identifiable without needing to be opened.

- Legibility: Use archival-grade, acid-free labels or permanent ink (no felt-tip pens that bleed).

- Consistency: Follow a standardized naming convention (e.g., Collection ID | Folder Range | Date Range).

- Visibility: Place labels on the short side of the box facing outward for quick scanning.

- Integrity: Check that labels aren't peeling or becoming brittle due to adhesive failure.

🗄️ 2. Shelving & Storage

The physical layout is your first line of defense against structural damage.

- Elevation: Ensure the bottom shelf is at least 4–6 inches off the floor to protect against minor flooding or cleaning splashes.

- Airflow: Leave a 2-inch gap between the back of the boxes and the wall to allow for air circulation.

- Stability: Store boxes upright and snugly; use spacers if a box is only half-full to prevent "slumping" or warping of the contents.

- Overhang: No part of the box should hang over the edge of the shelf (this causes "box fatigue" and potential falls).

🌡️ 3. Humidity & Temperature

The silent killers of paper are fluctuations in the air.

- Target Humidity: Maintain a Relative Humidity (RH) of 35% – 50%.

  - Too high: Risk of mold and pest infestations.

  - Too low: Paper becomes brittle and "shatters."

- Temperature: Ideally keep the area below 21°C (70°F). Consistency is more important than a specific number; avoid "sawtooth" fluctuations.

- Monitoring: Use a digital hygrometer to check for "micro-climates" near vents or windows.

🧤 4. Handling Practices

Human touch is often the most common source of wear and tear.

- Clean Hands: Wash and dry hands thoroughly. If handling photographs or metal artifacts within the files, use nitrile or clean cotton gloves.

- Support: Always support the bottom of a box when moving it; never pull it by the finger-hole or the lid.

- The "One-Folder" Rule: Only remove one folder at a time to ensure the filing order remains intact.

- Flat Surface: Only view files on a clean, flat, and dry surface—keep food and drinks in a separate "break zone."